ABSTRACT

The purpose of designing a plastic shredder is to reduce the growing problem of plastic waste is increasingly growing, it is because the plastic material is widely used both at home and in industry ladder, because the plastic material price mura and easily obtained. Therefore, plastic waste needs to be recycled by cutting-cut plastic into small flakes. Making it easier to recycle.

Model plastic shredder is very easy to operate. Have a size of 2212mm x 1574mm x 1410mm. So it does not have a large place to fine tune this plastic shredder. Plastic crusher machine is also equipped with a diesel motor. Where diesel motor serves as a power source to drive the shredder plastic to fit his needs. With motor power (P) 33 Hp = 24.25 kw at 2200 rpm motor rotation

In the destruction of the plastic shredder is equipped with 9 pieces of dynamic blades attached to the shaft by means of rotating and 2 static blades are attached to the front body and the rear body. With the blade material SKD 11 and tensile strength plastic (σ) 24 Mpa