Abstrak

Komposit Polimer Konduktif berpotensi digunakan sebagai bahan yang memiliki konduktivitas listrik dan kekerasan yang baik. Tujuan dari penelitian ini adalah meningkatkan nilai konduktivitas listrik dan kekerasan komposit polimer konduktif,  material komposit pada komposisi bahan pengisi konduktif 80% berat (wt.%) Dengan variasi ukuran dan komposisi bahan pengisi konduktif. Mendapatkan Parameter proses manufaktur material  komposit polimer konduktif. Eksperimental dengan mempariasikan komposisi pengisi kedua G75 (2,5%, 5%, 7,5%, 10%). Hasil dari penelitian ini adalah nilai Konduktivitas Listrik dengan spesimen dari kondisi berikut: (Komposisi 10%, suhu pemanasan 130˚C waktu pemanasan 120 menit) dengan nilai konduktivitas listrik 42,373 S.cm-1, dan Nilai kekerasan 94,866 HD.

Kata Kunci : Komposit, Grafit, Konduktivitas listrik, Kekerasan.

Abstract

Conductive Polymer Composite potentially  used as material which had electrical conductivity and good hardness. The purpose of this research is the increase the value of electric conductivity and hardness conductive polymer composites composite material on the a conductive filler material composition was 80% weight (wt%) With variations in size and composition of the conductive filler material. Calculated valve manufacturing process parameters conductive polymer composite materials. Experimentally with varieties the second filler composition G75 (2,5%, 5%, 7,5%, 10%). (Results of  this study was Electrical Conductivity value with speciments of  following conditions : The composition of 10%, the heating temperature 130C heating time for 120 minutes) with the value of electrical conductivity was 42,373 S.cm-1, and hardness value was 94,866 HD.

Keywords: Composite, Graphite, electrical conductivity, hardness.